# 12 Kinds of Fiber Identification Methods (1) - Microscopic Observation Method

Morphological Characteristics of Various Fibers

| Fiber Type | Longitudinal Morphology | Cross-Sectional Morphology |
| --- | --- | --- |
| Tencel Fiber | Smooth | Relatively regular round or oval, with skin-core layer |
| Modal Fiber | With 1 - 2 grooves longitudinally | Irregularly similar to kidney shape, relatively smooth, with skin-core |
| Soybean Fiber | With irregular grooves and island-like protrusions and depressions on the surface | Flat dumbbell-shaped and kidney-shaped |
| Bamboo Fiber | With grooves on the surface | Serrated, with skin-core layer |
| Chitin Fiber | With obvious grooves on the surface | Serrated at the edge, with obvious fine voids in the core layer |
| Viscose Fiber | With grooves on the surface | Serrated, with skin-core layer |
| Cotton Fiber | With natural twists | Kidney-shaped, with lumen |
| Silk | Smooth on the surface | Irregular triangle |
| Wool Fiber | With scales on the surface | Round |
| Polyester Fiber | Rod-shaped | Smooth and round on the surface |